Lcd keypad shield

Wiring:

Just pop the shield on

Code:

#include <LiquidCrystal.h>

LiquidCrystal lcd=LiquidCrystal(8,9,4,5,6,7);

const int ctrlp= A0;

int ctrl;

void setup() {

pinMode(ctrlp, INPUT);

lcd.begin(16,2);

Serial.begin(9600);

lcd.print("Press any button");

}

void loop() {

ctrl=analogRead(ctrlp);

Serial.println(ctrl);

//GIGA version has no select button

if(ctrl==1023) {

lcd.setCursor(0,1);

lcd.print("NONE  ");

lcd.clearWriteError();

}

if(ctrl>=0 && ctrl<50) {

lcd.setCursor(0,1);

lcd.print("RIGTH ");

lcd.clearWriteError();

}

if(ctrl>=150 && ctrl<400) {

lcd.setCursor(0,1);

lcd.print("UP  ");

lcd.clearWriteError();

}

if(ctrl>=400 && ctrl<500) {

lcd.setCursor(0,1);

lcd.print("DOWN ");

lcd.clearWriteError();

}

if(ctrl>=500 && ctrl<750) {

lcd.setCursor(0,1);

lcd.print("LEFT ");

lcd.clearWriteError();

}

}

Giga display shield touchscreen implement

Wiring:

Same

Code:

#include "Arduino_GigaDisplay_GFX.h"

#include "Arduino_GigaDisplayTouch.h"

Arduino_GigaDisplayTouch touchDetector;

GigaDisplay_GFX display;

#define WHITE 0xffff

#define BLACK 0x0000

#define screen_size_x 480

#define screen_size_y 800

int touch_x;

int touch_y;

//increase or decrease the sensitivity of the touch.

int trigger_sensitivity = 5;

bool switch_1;

int counter;

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

display.begin();

if (touchDetector.begin()) {

Serial.print("Touch controller init - OK");

} else {

Serial.print("Touch controller init - FAILED");

while (1)

;

}

}

void loop() {

uint8_t contacts;

GDTpoint_t points[5];

contacts = touchDetector.getTouchPoints(points);

if (contacts > 0) {

Serial.print("Contacts: ");

Serial.println(contacts);

counter++;

//record the x,y coordinates

for (uint8_t i = 0; i < contacts; i++) {

touch_x = points[i].x;

touch_y = points[i].y;

}

//as the display is 480x800, any time you touch the screen it will trigger

//the trigger sensitivity is set to "5".

if (touch_x < screen_size_x && touch_y < screen_size_y && counter > trigger_sensitivity) {

switch_1 = !switch_1;

Serial.println("switched");

changeSwitch();

delay(250);

}

}

}

void changeSwitch() {

if (switch_1) {

display.fillScreen(BLACK);

display.setTextColor(WHITE);

} else {

display.fillScreen(WHITE);

display.setTextColor(BLACK);

}

display.setCursor(50, screen_size_y/2);

display.setTextSize(5);

display.print("Switched");

counter = 0;

}

Camera with freeze button code:

#include "arducam_dvp.h"

#include "Arduino_H7_Video.h"

#include "dsi.h"

#include "SDRAM.h"

// This example only works with Greyscale cameras (due to the palette + resize&rotate algo)

#define ARDUCAM_CAMERA_OV767X

#if defined(ARDUCAM_CAMERA_OV767X)

#include "OV7670/ov767x.h"

//OV7670 ov767x;

OV7675 ov767x;

Camera cam(ov767x);

#define IMAGE_MODE CAMERA_RGB565

#endif

// The buffer used to capture the frame

FrameBuffer fb;

// The buffer used to rotate and resize the frame

FrameBuffer outfb;

// The buffer used to rotate and resize the frame

Arduino_H7_Video Display(800, 480, GigaDisplayShield);

const int b=2;

int s=0;

int t=0;

void blinkLED(uint32_t count = 0xFFFFFFFF)

{

pinMode(LED_BUILTIN, OUTPUT);

while (count--) {

digitalWrite(LED_BUILTIN, LOW);  // turn the LED on (HIGH is the voltage level)

delay(50);                       // wait for a second

digitalWrite(LED_BUILTIN, HIGH); // turn the LED off by making the voltage LOW

delay(50);                       // wait for a second

}

}

uint32_t palette[256];

void setup() {

pinMode(b,INPUT);

// Init the cam VGA, 30FPS

if (!cam.begin(CAMERA_R320x240, IMAGE_MODE, 30)) {

blinkLED();

}

// Setup the palette to convert 8 bit greyscale to 32bit greyscale

for (int i = 0; i < 256; i++) {

palette[i] = 0xFF000000 | (i << 16) | (i << 8) | i;

}

Display.begin();

if (IMAGE_MODE == CAMERA_GRAYSCALE) {

dsi_configueCLUT((uint32_t*)palette);

}

outfb.setBuffer((uint8_t*)SDRAM.malloc(1024 * 1024));

// clear the display (gives a nice black background)

dsi_lcdClear(0);

dsi_drawCurrentFrameBuffer();

dsi_lcdClear(0);

dsi_drawCurrentFrameBuffer();

Serial.begin(115200);

}

#define HTONS(x)    (((x >> 8) & 0x00FF) | ((x << 8) & 0xFF00))

void loop() {

while(digitalRead(b)==HIGH) {

}

// Grab frame and write to another framebuffer

if (cam.grabFrame(fb, 3000) == 0) {

// double the resolution and transpose (rotate by 90 degrees) in the same step

// this only works if the camera feed is 320x240 and the area where we want to display is 640x480

for (int i = 0; i < 320; i++) {

for (int j = 0; j < 240; j++) {

if (IMAGE_MODE == CAMERA_GRAYSCALE) {

((uint8_t*)outfb.getBuffer())[j * 2 + (i * 2) * 480] = ((uint8_t*)fb.getBuffer())[i + j * 320];

((uint8_t*)outfb.getBuffer())[j * 2 + (i * 2) * 480 + 1] = ((uint8_t*)fb.getBuffer())[i + j * 320];

((uint8_t*)outfb.getBuffer())[j * 2 + (i * 2 + 1) * 480] = ((uint8_t*)fb.getBuffer())[i + j * 320];

((uint8_t*)outfb.getBuffer())[j * 2 + (i * 2 + 1) * 480 + 1] = ((uint8_t*)fb.getBuffer())[i + j * 320];

} else {

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2) * 480] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2) * 480 + 1] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2 + 1) * 480] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2 + 1) * 480 + 1] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

}

}

}

dsi_lcdDrawImage((void*)outfb.getBuffer(), (void*)dsi_getCurrentFrameBuffer(), 480, 640, IMAGE_MODE == CAMERA_GRAYSCALE ? DMA2D_INPUT_L8 : DMA2D_INPUT_RGB565);

dsi_drawCurrentFrameBuffer();

} else {

blinkLED(20);

}

}

Litterally same thing but cool logo

#include "arducam_dvp.h"

#include "Arduino_H7_Video.h"

#include "dsi.h"

#include "SDRAM.h"

#include "Arduino_GigaDisplay_GFX.h"

// This example only works with Greyscale cameras (due to the palette + resize&rotate algo)

#define ARDUCAM_CAMERA_OV767X

#include "OV7670/ov767x.h"

OV7670 ov767x;

//OV7675 ov767x;

Camera cam(ov767x);

#define IMAGE_MODE CAMERA_RGB565

//for smasnugfon69 mark

GigaDisplay_GFX display;

// The buffer used to capture the frame

FrameBuffer fb;

// The buffer used to rotate and resize the frame

FrameBuffer outfb;

// The buffer used to rotate and resize the frame

Arduino_H7_Video Display(800, 480, GigaDisplayShield);

const int b=2;

int n=0;

void blinkLED(uint32_t count = 0xFFFFFFFF)

{

pinMode(LED_BUILTIN, OUTPUT);

while (count--) {

digitalWrite(LED_BUILTIN, LOW);  // turn the LED on (HIGH is the voltage level)

delay(50);                       // wait for a second

digitalWrite(LED_BUILTIN, HIGH); // turn the LED off by making the voltage LOW

delay(50);                       // wait for a second

}

}

uint32_t palette[256];

void logoprint() {

display.setCursor(10,740); //x,y

display.setTextSize(5); //adjust text size

display.setTextColor(0x0000);//if text does not appear simply comment this line

display.print("smasnugfon69");

}

void setup() {

pinMode(b,INPUT);

display.begin(); //init library

// Init the cam VGA, 30FPS

if (!cam.begin(CAMERA_R320x240, IMAGE_MODE, 30)) {

blinkLED();

}

//begin display

Display.begin();

outfb.setBuffer((uint8_t*)SDRAM.malloc(1024 * 1024));

// clear the display (gives a nice black background)

dsi_lcdClear(0);

dsi_drawCurrentFrameBuffer();

dsi_lcdClear(0);

dsi_drawCurrentFrameBuffer();

Serial.begin(115200);

}

#define HTONS(x)    (((x >> 8) & 0x00FF) | ((x << 8) & 0xFF00))

void loop() {

while(digitalRead(b)==HIGH) {

Serial.println("freeze");

}

if(digitalRead(b)==LOW) {

Serial.println("out of while");

}

// Grab frame and write to another framebuffer

if (cam.grabFrame(fb, 3000) == 0) {

// double the resolution and transpose (rotate by 90 degrees) in the same step

// this only works if the camera feed is 320x240 and the area where we want to display is 640x480

for (int i = 0; i < 320; i++) {

for (int j = 0; j < 240; j++) {

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2) * 480] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2) * 480 + 1] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2 + 1) * 480] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

((uint16_t*)outfb.getBuffer())[j * 2 + (i * 2 + 1) * 480 + 1] = HTONS(((uint16_t*)fb.getBuffer())[i + j * 320]);

}

}

dsi_lcdDrawImage((void*)outfb.getBuffer(), (void*)dsi_getCurrentFrameBuffer(), 480, 640, IMAGE_MODE == CAMERA_GRAYSCALE ? DMA2D_INPUT_L8 : DMA2D_INPUT_RGB565);

dsi_drawCurrentFrameBuffer();

} else {

blinkLED(20);

}

}

Camera v2 code:

#include <arducam_dvp.h>

#include <Arduino_H7_Video.h>

#include <dsi.h>

#include <SDRAM.h>

#define ARDUCAM_CAMERA_OV767x

#include "OV7670/ov767x.h"

OV7670 ov767x;

Camera cam(ov767x);

#define IMAGE_MODE CAMERA_RGB565

FrameBuffer CapFB; // The buffer used to capture the frame

FrameBuffer OutFB; // The buffer used to rotate and resize the frame

Arduino_H7_Video Display(800, 480, GigaDisplayShield);

void blinkLED(uint32_t count = 0xFFFFFFFF)

{

pinMode(LED_BUILTIN, OUTPUT);

while (count--)

{

digitalWrite(LED_BUILTIN, LOW);  // turn the LED on (HIGH is the voltage level)

delay(50);                       // wait for a second

digitalWrite(LED_BUILTIN, HIGH); // turn the LED off by making the voltage LOW

delay(50);                       // wait for a second

}

}

uint32_t palette[256];

void setup()

{

if (!cam.begin(CAMERA_R320x240, IMAGE_MODE, 30)) // Init the cam QVGA, 30FPS

{

blinkLED();

}

Display.begin();

OutFB.setBuffer((uint8_t*)SDRAM.malloc(1024 * 1024));

// clear the display (gives a nice black background)

dsi_lcdClear(0);

dsi_drawCurrentFrameBuffer();

dsi_lcdClear(0);

dsi_drawCurrentFrameBuffer();

}

#define HTONS(x)    (((x >> 8) & 0x00FF) | ((x << 8) & 0xFF00))

void loop()

{

// Grab frame and write to another framebuffer

if (cam.grabFrame(CapFB, 3000) == 0)

{

for (int i = 0; i < 320; i++)

{

for (int j = 0; j < 240; j++)

{

//((uint16_t*)OutFB.getBuffer())[j + i * 240] = HTONS(((uint16_t*)CapFB.getBuffer())[i + j * 320]);

((uint16_t*)OutFB.getBuffer())[j + i * 240] = HTONS(((uint16_t*)CapFB.getBuffer())[j + i * 240]);

((uint16_t*)OutFB.getBuffer())[i + j * 320] = HTONS(((uint16_t*)CapFB.getBuffer())[i + j * 320]);

//((uint16_t*)OutFB.getBuffer())[i + j * 320] = HTONS(((uint16_t*)CapFB.getBuffer())[j + i * 240]);

}

}

//dsi_lcdDrawImage((void*)OutFB.getBuffer(), (void*)dsi_getCurrentFrameBuffer(), 240, 320, IMAGE_MODE == CAMERA_GRAYSCALE ? DMA2D_INPUT_L8 : DMA2D_INPUT_RGB565);

dsi_lcdDrawImage((void*)OutFB.getBuffer(), (void*)dsi_getCurrentFrameBuffer(), 320, 240, IMAGE_MODE == CAMERA_GRAYSCALE ? DMA2D_INPUT_L8 : DMA2D_INPUT_RGB565);

dsi_drawCurrentFrameBuffer();

}

else

{

blinkLED(20);

}

}

Moving ball code:

#include "Arduino_GigaDisplay_GFX.h"

GigaDisplay_GFX display;

const int right=52;

const int left=53;

const int up=50;

const int down=51;

bool r;

bool l;

bool u;

bool d;

int x=240;

int y=400;

#define WHITE 0xffff

#define BLACK 0x0000

#define RED 0xf800

void setup() {

display.begin();

display.fillScreen(BLACK);

display.fillCircle(x,y,15,RED);

pinMode(down,INPUT);

pinMode(up,INPUT);

pinMode(right,INPUT);

pinMode(left,INPUT);

Serial.begin(115200);

}

void getbuttonfeedback() {

if(digitalRead(right)==HIGH) {

r=true;

Serial.print(r);

}

else{

r=false;

Serial.print(r);

}

if(digitalRead(left)==HIGH) {

l=true;

Serial.print(l);

}

else{

l=false;

Serial.print(l);

}

if(digitalRead(up)==HIGH) {

u=true;

Serial.print(u);

}

else{

u=false;

Serial.print(u);

}

if(digitalRead(down)==HIGH) {

d=true;

Serial.println(d);

}

else{

d=false;

Serial.println(d);

}

}

void loop() {

getbuttonfeedback();

display.fillCircle(x,y,15,RED);

if(y>=800) {

y=1;

}

if(y<=0) {

y=799;

}

if(x>=480) {

x=1;

}

if(x<=0) {

x=479;

}

if(r) {

display.fillScreen(BLACK);

y=y+5;

Serial.println("rmove");

}

else if(!r) {}

if(l) {

display.fillScreen(BLACK);

y=y-5;

Serial.println("lmove");

}

else if(!l) {}

if(u) {

display.fillScreen(BLACK);

x=x+5;

Serial.println("umove");

}

else if(!u) {}

if(d) {

display.fillScreen(BLACK);

x=x-5;

Serial.println("dmove");

}

else if(!d) {}

}

5 games in one console code:

#include "Arduino_GigaDisplay_GFX.h"

GigaDisplay_GFX display;

const int right=52;

const int left=53;

const int up=50;

const int down=51;

const int game=A0;

int gamesel=0;

bool r;

bool l;

bool u;

bool d;

bool tr;

int x=240;

int y=400;

#define WHITE 0xffff

#define BLACK 0x0000

#define RED 0xf800

#define BLUE 0x001f

#define ORANGE 0xfc40

void setup() {

display.begin();

display.fillScreen(BLACK);

display.fillCircle(x,y,15,RED);

pinMode(down,INPUT);

pinMode(up,INPUT);

pinMode(right,INPUT);

pinMode(left,INPUT);

pinMode(game,INPUT);

Serial.begin(115200);

}

void getbuttonfeedback() {

if(digitalRead(right)==HIGH) {

r=true;

//Serial.print(r);

}

else{

r=false;

//Serial.print(r);

}

if(digitalRead(left)==HIGH) {

l=true;

//Serial.print(l);

}

else{

l=false;

//Serial.print(l);

}

if(digitalRead(up)==HIGH) {

u=true;

//Serial.print(u);

}

else{

u=false;

//Serial.print(u);

}

if(digitalRead(down)==HIGH) {

d=true;

//Serial.println(d);

}

else{

d=false;

//Serial.println(d);

}

}

void gameswitch() {

gamesel=analogRead(game);

Serial.println(gamesel);

}

void drawrandomcircles(int j) {

for(int i=0;i<j;i++) {

int yc=random(0,480);

int xc=random(0,800);

int rc=random(20,100);

display.fillCircle(yc,xc,rc,WHITE);

}

}

void loop() {

gameswitch();

//simple moving ball  ///////////////////////////////////////////////////////////////////////////////////

while(gamesel<204) {

gameswitch();

getbuttonfeedback();

display.fillCircle(x,y,15,WHITE);

if(y>=800) {

y=1;

}

if(y<=0) {

y=799;

}

if(x>=480) {

x=1;

}

if(x<=0) {

x=479;

}

if(r) {

display.fillScreen(BLACK);

y=y+5;

Serial.println("rmove");

}

else if(!r) {}

if(l) {

display.fillScreen(BLACK);

y=y-5;

Serial.println("lmove");

}

else if(!l) {}

if(u) {

display.fillScreen(BLACK);

x=x+5;

Serial.println("umove");

}

else if(!u) {}

if(d) {

display.fillScreen(BLACK);

x=x-5;

Serial.println("dmove");

}

else if(!d) {}

}

//draw  ////////////////////////////////////////////////////////////////////////////

while(gamesel>=204 && gamesel<409) {

gameswitch();

getbuttonfeedback();

display.fillCircle(x,y,15,BLUE);

if(y>=800) {

y=1;

}

if(y<=0) {

y=799;

}

if(x>=480) {

x=1;

}

if(x<=0) {

x=479;

}

if(r) {

y=y+5;

delay(10);

Serial.println("rmove");

}

else if(!r) {}

if(l) {

y=y-5;

delay(10);

Serial.println("lmove");

}

else if(!l) {}

if(u) {

x=x+5;

delay(10);

Serial.println("umove");

}

else if(!u) {}

if(d) {

x=x-5;

delay(10);

Serial.println("dmove");

}

else if(!d) {}

}

//avoid white balls  /////////////////////////////////////////////////////////////////////

while(gamesel>=409 && gamesel<614) {

gameswitch();

getbuttonfeedback();

Serial.println(digitalRead(game));

display.fillCircle(x,y,15,RED);

if(y>=800) {

display.fillScreen(BLACK);

drawrandomcircles(15);

y=1;

}

if(y<=0) {

display.fillScreen(BLACK);

drawrandomcircles(15);

y=799;

}

if(x>=480) {

display.fillScreen(BLACK);

drawrandomcircles(15);

x=1;

}

if(x<=0) {

display.fillScreen(BLACK);

drawrandomcircles(15);

x=479;

}

if(r) {

y=y+10;

delay(10);

Serial.println("rmove");

}

else if(!r) {}

if(l) {

y=y-10;

delay(10);

Serial.println("lmove");

}

else if(!l) {}

if(u) {

x=x+10;

delay(10);

Serial.println("umove");

}

else if(!u) {}

if(d) {

x=x-10;

delay(10);

Serial.println("dmove");

}

else if(!d) {}

}

// gd normal wave  /////////////////////////////////////////////////////////////////////////////

while(gamesel>=614 && gamesel<819) {

gameswitch();

getbuttonfeedback();

display.fillCircle(x,y,15,ORANGE);

y=y+10;

delay(10);

if(y>=800) {

display.fillScreen(BLACK);

drawrandomcircles(10);

y=1;

}

if(y<=0) {

display.fillScreen(BLACK);

drawrandomcircles(10);

y=799;

}

if(x>=480) {

x=1;

}

if(x<=0) {

x=479;

}

if(r) {

x=x+10;

Serial.println("up");

}

else if(!r) {

x=x-10;

Serial.println("down");

}

if(l) {}

else if(!l) {}

if(u) {}

else if(!u) {}

if(d) {}

else if(!d) {}

}

// gd mini wave  /////////////////////////////////////////////////////////////////////////////

while(gamesel>819) {

gameswitch();

getbuttonfeedback();

display.fillCircle(x,y,15,ORANGE);

y=y+10;

delay(10);

if(y>=800) {

display.fillScreen(BLACK);

drawrandomcircles(5);

y=1;

}

if(y<=0) {

display.fillScreen(BLACK);

drawrandomcircles(5);

y=799;

}

if(x>=480) {

x=1;

}

if(x<=0) {

x=479;

}

if(r) {

x=x+20;

Serial.println("up");

}

else if(!r) {

x=x-20;

Serial.println("down");

}

if(l) {}

else if(!l) {}

if(u) {}

else if(!u) {}

if(d) {}

else if(!d) {}

}

}

Calculator

Wiring:

Run the keypad pins through a logic level convertor. On the keypad, the 3 rightmost pins are the column pins.

Code:

//ALPHA CALCULATOR

/*credits to copilot for helping me out on this one

comments that have a space going on (// Hi) are copilot's comments*/

#include <Keypad.h>

#include <Arduino_GigaDisplay_GFX.h>

GigaDisplay_GFX s;

int a = 0;

int b = 0;

int r = 0;

char c;

bool o;

//byte because it occupies less space than an int

const byte numRows = 4;

const byte numCols = 3;

char keymap[numRows][numCols] = {

{'1', '2', '3'},

{'4', '5', '6'},

{'7', '8', '9'},

{'*', '0', '#'}

};

byte rowPins[numRows] = {30, 32, 34, 36};

byte colPins[numCols] = {31, 33, 35};

Keypad pad = Keypad(makeKeymap(keymap), rowPins, colPins, numRows, numCols);

char presd;  // Use char to store key presses

void setup() {

s.begin();

Serial.begin(9600);

sreset();

}

void sreset() {

s.fillScreen(0x0000);

s.setTextColor(0xffff);

s.setCursor(0,50);

s.setTextSize(4);

}

void loop() {

while (true) {  // Infinite loop to keep prompting for numbers

a = 0;

b = 0;

c = 0;

Serial.println("Insert first number:");

s.print("Insert first number");

o = true;

while (o == true) {

delay(100);

getkey();

if (presd == '#') {  // Assuming '#' represents a specific key like Enter //yea

o = false;

} else if (presd == '*') {  // Assuming '*' represents a backspace key //mhm

a = (a - (a % 10)) / 10;

Serial.println(a);

sreset();

s.print(a);

} else if (presd != NO_KEY) {  // Check if a valid key is pressed //isnt nokey a char?

int num = presd - '0';  // Convert char to int //oh.

a = a * 10 + num;

Serial.println(a);

sreset();

s.print(a);

}

}

Serial.println("Insert second number:");

sreset();

s.print("Insert second number");

o = true;

while (o == true) {

delay(100);

getkey();

if (presd == '#') {  // Assuming '#' represents a specific key like Enter

o = false;

} else if (presd == '*') {  // Assuming '*' represents a backspace key

b = (b - (b % 10)) / 10;

Serial.println(b);

sreset();

s.print(b);

} else if (presd != NO_KEY) {  // Check if a valid key is pressed

int num = presd - '0';  // Convert char to int

b = b * 10 + num;

Serial.println(b);

sreset();

s.print(b);

}

}

// Perform a simple calculation (addition, in this case)

//nah how bout operation of your choice

Serial.println("Operation");

sreset();

s.print("Operation");

o = true;

while (o == true) {

delay(100);

getkey();

if (presd == '#') {

o = false;

} else if (presd == '*') {

c='0';

Serial.println(c);

sreset();

s.print(c);

} else if (presd=='1') {

c='+';

Serial.println(c);

sreset();

s.print(c);

} else if (presd=='2') {

c='-';

Serial.println(c);

sreset();

s.print(c);

} else if (presd=='3') {

c='*';

Serial.println(c);

sreset();

s.print(c);

} else if (presd=='4') {

c='/';

Serial.println(c);

sreset();

s.print(c);

} else if (presd=='5') {

c='%';

Serial.println(c);

sreset();

s.print(c);

}

}

//doing the operations

if(c=='+') {

r=a+b;

}

if(c=='-') {

r=a-b;

}

if(c=='*') {

r=a*b;

}

if(c=='/') {

r=a/b;

}

if(c=='%') {

r=a%b;

}

//show result

Serial.print("Answer: "); Serial.println(r);

sreset();

s.print("Answer: "); s.println(r);

// Automatically start over to prompt for new numbers

}

}

void getkey() {

presd = pad.getKey();  // Use the global variable presd

}

WaSense (soil humidity sensor)

Wiring:

Connect the lcd keypad shield and then a piezo, rgb led and humidity sensor

Code:

#include <LiquidCrystal.h>

LiquidCrystal d=LiquidCrystal(8,9,4,5,6,7);

int i; int p;

const int s=A1;

const int r=22;

const int g=23;

const int b=24;

const int piezo=25;

void setup() {

pinMode(r,OUTPUT);

pinMode(g,OUTPUT);

pinMode(b,OUTPUT);

pinMode(s,INPUT);

d.begin(16,2);

Serial.begin(9600);

}

void loop() {

i=analogRead(A1);

p=map(i,0,1024,100,0);

String v=String(p);

d.clear(); d.write("Humidity:");

d.setCursor(0,1); d.print(v);

d.setCursor(3,1); d.print("%");

d.setCursor(12,1); d.print("50");

d.setCursor(15,1); d.print("%");

//Serial.println(p);

if(p<45) {

digitalWrite(r, HIGH);

digitalWrite(g, LOW);

digitalWrite(b, LOW);

tone(piezo,500);

Serial.println("r");

}

else if(35<=p && p<45) {

digitalWrite(r, HIGH);

digitalWrite(g, HIGH);

digitalWrite(b, LOW);

noTone(piezo);

Serial.println("rg");

}

else if(45<=p && p<=55) {

digitalWrite(r, LOW);

digitalWrite(g, HIGH);

digitalWrite(b, LOW);

noTone(piezo);

Serial.println("g");

}

else if(55<p && p<=60) {

digitalWrite(r, LOW);

digitalWrite(g, HIGH);

digitalWrite(b, HIGH);

noTone(piezo);

Serial.println("gb");

}

else if(60<p) {

digitalWrite(r, LOW);

digitalWrite(g, LOW);

digitalWrite(b, HIGH);

noTone(piezo);

Serial.println("b");

}

delay(50);

}

Giga WiFi access

Wiring:

Attach the antenna

Code:

#include <SPI.h>

#include <WiFi.h>

#include "arduino_secrets.h"

// Replace with your actual credentials in arduino_secrets.h

char ssid[] = SECRET_SSID;

char pass[] = SECRET_PASS;

int status = WL_IDLE_STATUS;

char server[] = "speedtest.tele2.net"; // Public test server

WiFiClient client;

void setup() {

Serial.begin(9600);

while (!Serial);

if (WiFi.status() == WL_NO_MODULE) {

Serial.println("WiFi module not found!");

while (true);

}

while (status != WL_CONNECTED) {

Serial.print("Connecting to SSID: ");

Serial.println(ssid);

status = WiFi.begin(ssid, pass);

delay(5000);

}

Serial.println("Connected to network");

printWifiData();

}

void loop() {

downloadTestFile();

printCurrentNet();

delay(100); // Short delay to keep traffic flowing

}

void downloadTestFile() {

Serial.println("Starting download...");

if (client.connect(server, 80)) {

client.println("GET /1MB.zip HTTP/1.1");

client.println("Host: ipv4.download.thinkbroadband.com");

client.println("User-Agent: Arduino");

client.println("Connection: close");

client.println();

unsigned long bytesRead = 0;

unsigned long startTime = millis();

unsigned long timeout = 30000; // 30 seconds max

while (client.connected()) {

if (client.available()) {

char c = client.read();

bytesRead++;

if (bytesRead <= 100) Serial.write(c); // Print first 100 bytes

}

}

Serial.println();

Serial.print("Downloaded bytes: ");

Serial.println(bytesRead);

client.stop();

} else {

Serial.println("Connection failed");

}

}

void printWifiData() {

IPAddress ip = WiFi.localIP();

Serial.print("IP Address: ");

Serial.println(ip);

byte mac[6];

WiFi.macAddress(mac);

Serial.print("MAC address: ");

printMacAddress(mac);

}

void printCurrentNet() {

Serial.print("SSID: ");

Serial.println(WiFi.SSID());

byte bssid[6];

WiFi.BSSID(bssid);

Serial.print("BSSID: ");

printMacAddress(bssid);

long rssi = WiFi.RSSI();

Serial.print("Signal strength (RSSI): ");

Serial.println(rssi);

byte encryption = WiFi.encryptionType();

Serial.print("Encryption Type: ");

Serial.println(encryption, HEX);

Serial.println(); // Empty line for spacing

}

void printMacAddress(byte mac[]) {

for (int i = 5; i >= 0; i--) {

if (mac[i] < 16) Serial.print("0");

Serial.print(mac[i], HEX);

if (i > 0) Serial.print(":");

}

Serial.println();

}

arduino_secrets.h

//arduino_secrets.h header file

//#define SECRET_SSID "Z Flip3"

//#define SECRET_PASS "00000000"// my hotspot

#define SECRET_SSID "Mastery"

#define SECRET_PASS "MElectroniX1"

Detailed WiFi access

Wiring:

Antenna

Code:

#include <SPI.h>

#include <WiFi.h>

#include "arduino_secrets.h" // Contains SECRET_SSID and SECRET_PASS

char ssid[] = SECRET_SSID;

char pass[] = SECRET_PASS;

int status = WL_IDLE_STATUS;

char server[] = "speedtest.tele2.net"; // Public test server

WiFiClient client;

void setup() {

Serial.begin(115200);

while (!Serial);

if (WiFi.status() == WL_NO_MODULE) {

Serial.println("WiFi module not found!");

while (true);

}

while (status != WL_CONNECTED) {

Serial.print("Connecting to SSID: ");

Serial.println(ssid);

status = WiFi.begin(ssid, pass);

delay(5000);

}

Serial.println("✅ Connected to network");

printWifiData();

}

void loop() {

static bool downloaded = false;

if (!downloaded) {

downloadTestFile();

downloaded = true;

}

}

void downloadTestFile() {

Serial.println("🚀 Starting download...");

client.setTimeout(60000); // Increase timeout for large file

if (client.connect(server, 80)) {

client.println("GET /1MB.zip HTTP/1.1");

client.println("Host: speedtest.tele2.net");

client.println("User-Agent: Arduino-GIGA");

client.println("Connection: close");

client.println();

unsigned long bytesRead = 0;

unsigned long startTime = millis();

char buffer[128];

while (client.connected()) {

int len = client.readBytes(buffer, sizeof(buffer));

bytesRead += len;

// Print first 100 bytes for verification

for (int i = 0; i < len && bytesRead <= 100; i++) {

Serial.write(buffer[i]);

}

}

unsigned long duration = millis() - startTime;

float speedKbps = (bytesRead * 8.0) / (duration / 1000.0) / 1000.0;

Serial.println();

Serial.print("📦 Downloaded bytes: ");

Serial.println(bytesRead);

Serial.print("⏱️ Duration: ");

Serial.print(duration);

Serial.println(" ms");

Serial.print("⚡ Speed: ");

Serial.print(speedKbps);

Serial.println(" kbps");

client.stop();

} else {

Serial.println("❌ Connection failed");

}

}

void printWifiData() {

IPAddress ip = WiFi.localIP();

Serial.print("IP Address: ");

Serial.println(ip);

byte mac[6];

WiFi.macAddress(mac);

Serial.print("MAC address: ");

printMacAddress(mac);

}

void printMacAddress(byte mac[]) {

for (int i = 5; i >= 0; i--) {

if (mac[i] < 16) Serial.print("0");

Serial.print(mac[i], HEX);

if (i > 0) Serial.print(":");

}

Serial.println();

}

arduino_secrets.h

//arduino_secrets.h header file

//#define SECRET_SSID "Z Flip3"

//#define SECRET_PASS "00000000"// my hotspot

#define SECRET_SSID "Mastery"

#define SECRET_PASS "MElectroniX1"